MURANG’A SOUTH SUB COUNTY

HOME SCIENCE

CLOTHING CONSTRUCTION

FORM FOUR 2025 OPENER EXAM TERM 3

P441/ 2 MARKING SCHEME

|  | PROCESS | MAX SCORE | SCORE | REMARKS |
| --- | --- | --- | --- | --- |
| 1 a b c d | PRESENTATION Work well pressed (1) and folded (1/2) Label (1) firmly fixed (1/2) on single (1/2) fabric without concealing details (1/2) Pins (1/2) needles (1/2) tacking (1/2) unnecessary threads (1/2) removed Made for the left half (1) | 11/2 21/2 11/2 1 |  |  |
|  | Sub – Total | 61/2 |  |  |
| 2 a b c d e f g | CUTTING OUT All eight pieces cut out (4) Smooth cutting of bodice CF (1) on grain (1) Smooth cutting of bodice CB (1) on grain (1) Smooth cutting of skirt CF (1) skirt front cut on bias (1) Smooth cutting of skirt CB (1) skirt back cut on bias (1) Smooth cutting of skirt lower edge 1 × 2 (2) Smooth cutting of sleeve lower edge (1) | 4 2 2 2 2 2 1 |  |  |
|  | Sub – Total | 15 |  |  |
| 3 a b c d e | MAKING OF THE DART AT THE BODICE FRONT Straight stitchery of dart (1) tapering to a point on the fold (1/2) and secured at the end (1) Correct length of dart 7.5 cm to within 2 mm (7.3 cm – 7.8 cm) (1) Correct width of dart 1.4 cm to within 2 mm (1.2 cm – 1.6 cm) (1) Dart pressed towards the CF (1) Dart flat on WS (1/2) and RS (1/2) | 21/2 1 1 1 1 |  |  |
|  | Sub – Total | 61/2 |  |  |
| 4 a b c d e f g | MAKING THE DOUBLE STITCHED SEAM AT THE SHOULDER (Award zero if not double stitched seam) First row of stitchery made with straight stitches (1) Second row of stitchery made with straight stitches (1) close to the fold (1) with raw edges well locked under (1) Seam made on RS (1) Correct size of seam 6mm to within 2mm (4mm – 8mm) (1) Evenness of the seam (1) Seam turned to the back side (1) Seam flat on RS (1) and WS (1) | 1 3 1 1 1 1 2 |  |  |
|  | Sub – Total | 14 |  |  |
| 5 a b c d e a b c d e f | MAKING THE SEAM AT THE SIDE OF THE BODICE AND SKIRT USING OPEN SEAM (Award zero if not open seam) Bodice Seam made with straight stitchery (1) Seam neatened (2) Correct size of the seam 1cm to within 2mm (0.8cm – 1.2cm) (1) Evenness of the seam (1) Seam pressed open and flat at underarm RS (1) WS (1) at waist RS (1) WS (1) Skirt Seam made with straight stitchery (1) Seam neatened (2) Correct size of seam 1cm to within 2mm (0.8cm – 1.2cm) (1) Evenness of the seam (1) Seam open and flat at the waist on RS (1) and WS (1) Front and back skirt flash at lower edge (1/2) | 1 2 1 1 4 1 2 1 1 2 ½ |  |  |
|  | Sub – Total | 161/2 |  |  |
| 6 a b c d e | ATTACHING OF THE BODICE PIECES TO THE SKIRT PIECES USING PLAIN SEAM AND NEATENEING HALF OF THE BACK SECTION USING LOOP STITCHES (Award zero if not plain seam) Seam joined with straight stitchery (1) Neatened section trimmed to 1cm to within 2mm (0.8cm – 1.2cm) (1) Untrimmed section notches (1/2) matched (1/2) and bodice Skirt flash at CF (1/2) CB (1/2) and side seam (1/2) Loop stitches uniform in length (1/2) well distributed (1/2) correct tension (1/2) and secured at the beginning (1/2) and at the end (1/2) | 1 1 1 11/2 21/2 |  |  |
|  | Sub – Total | 7 |  |  |
| 7 a b c d e f g h | PREPARATION AND ATTACHMENT OF THE COLLAR Collar joined with smooth stitchery (1) seam trimmed (1) notched (1) (mark by impression) and knife edged (1) Collar attached to the dress with smooth stitchery (1) trimmed (1) and notched (1) (mark by impression) Crossway well tucked under (1/2) machine stitched (1/2) smooth stitchery (1/2) close to the fold (1/2) and flat WS (1/2) RS (1/2) Correct width of crossway strip 6mm to within 2mm (4mm – 8mm) (1) Correct position of collar 1cm from CB to within 2mm (0.8cm – 1.2cm) and at flash with CF (1) Correct size (1) and shape (1) of collar (Superimpose to get size + shape) Collar lies flat on dress (1) Crossway strip flash with bodice at CF 1/2 × 2 (1) | 4 3 3 1 1 2 1 1 |  |  |
|  | Sub – Total | 16 |  |  |
| 8 a b c d e f g h i j | PREPARATION AND ATTACHING THE SLEEVE (Award zero if underarm is not open seam a - e) Underarm made with straight stitchery (1) Seam neatened (1) Correct size of the seam 1cm to within 2mm (0.8cm – 1.2cm) (1) Evenness of the seam (1) Seam open and flat at the armhole RS (1) WS (1) Good hang of the sleeve (1) Sleeve attached with smooth stitchery (1) Notches 1/2 × 2 (1) matched (1/2) Gathers (1/2) well distributed (1/2) at the crown (1/2) Underarm and side seams flash (1/2) | 1 1 1 1 2 1 1 11/2 11/2 ½ |  |  |
|  |  | 111/2 |  |  |
|  | Total Score | 90 |  |  |
|  | Get the final mark by dividing the total by 2 (90 ÷ 2) |  |  |  |